TABLE OF CONTENTS

i2csensors.library/--------------Introduction----------------

i2csensors.library/--------------Installation----------------

i2csensors.library/--------------Configuration---------------

i2csensors.library/---------Configuration_keywords-----------

i2csensors.library/i2c_SensorNum

i2csensors.library/i2c_ReadSensor

i2csensors.library/i2c_Obtain

i2csensors.library/i2c_Release

i2csensors.library/i2c_Send

i2csensors.library/i2c_Receive

i2csensors.library/i2c_SensorNum               i2csensors.library/i2c_SensorNum

NAME

i2c_SensorNum - determine number of Sensors of specific types

SYNOPSIS

NSensors = i2c_SensorNum( type )

D0                        D0

LONG i2c_SensorNum(LONG);

FUNCTION

The library parses the configuration files in Devs:Sensors/ in its

initialization phase and provides a relatively generic interface

to applications.

This function obtains the number of defined sensors for a given type.

The initial pre-defined types are I2C_VOLTAGE, I2C_CURRENT, I2C_TEMP,

I2C_FAN and I2C_PRESSURE. Please see libraries/i2cclass.h for additional

future extensions.

This function may not be called from interrupt code.

DESCRIPTION

Typical applications group their presented numbers into different

classes. Following that concept, the sensor reading in this library

is by type. The output of this function is purely informative.

It must be noted that the user might have specified sensors in the

configuration files that are not phyically present or active on the

bus. Therefore, errors need to be expected.

INPUTS

type     - one of I2C_VOLTAGE, I2C_CURRENT, I2C_TEMP, ...

RESULTS

NSensors - Number of sensors. If none are specified for the current

type, the returned value is zero.

BUGS

SEE ALSO

libraries/i2cclass.h, libraries/i2csensors.h, i2c_ReadSensor()

i2csensors.library/i2c_ReadSensor              i2csensors.library/i2c_ReadSensor

NAME

i2c_ReadSensor - read from a sensor

SYNOPSIS

Value = i2c_ReadSensor(type, index, unitstring, namestring )

D0                     D0    D1     A1          A2

LONG i2c_ReadSensor(LONG, LONG, BYTE**, BYTE** );

FUNCTION

This function reads from a sensor and returns the measured value

along with the user-defined title of the sensor and the measurement

unit.

This function may not be called from interrupt code.

DESCRIPTION

Typical applications group their presented numbers into different

classes. Following that concept, the sensor reading in this library

is by type and index. Please see i2c_SensorNum for more information

regarding the defined types.

The index argument starts with index 0 for the first sensor and is

to be incremented by 1 for each subsequent sensor.

The two string arguments are provided to return the proper

name and unit strings by the library.

It must be noted that the user might have specified sensors in the

configuration files that are not phyically present or active on the

bus. Therefore, errors need to be expected.

By definition, an error may be detected by the case that both

unitstring and namestring are zero after this call.

The returned Value is specified as 16.16 fixed point number. If desired,

a conversion to floating point could be obtained by multiplying with

1/65536.

INPUTS

type     - one of I2C_VOLTAGE, I2C_CURRENT, I2C_TEMP, ...

index    - index (0, 1, 2, ...)

unitstring - storage for the returned measurement unit string pointer

namestring - storage for the returned sensor name string pointer

RESULTS

Value    - measured value as 16.16 fixed point number

BUGS

SEE ALSO

libraries/i2cclass.h, libraries/i2csensors.h, i2c_SensorNum()

i2csensors.library/i2c_Obtain             i2csensors.library/i2c_Obtain

NAME

i2c_Obtain - lock i2c bus for a sequence of read/write operations

SYNOPSIS

RESULT = i2c_Obtain( i2cAddress )

D0                   D0

LONG i2c_Obtain(LONG);

FUNCTION

Serialize I2C bus access by blocking competing tasks until

a read/write cycle is complete.

This function may not be called from interrupt code.

DESCRIPTION

One potential issue with the i2c.library is that competing

tasks may interfere with each other's read/write operations

on the bus, especially when a task switch occurs after the

prologue of an operation.

This call was added to ensure that each series of read/write

operations can be carried out without interference.

Currently, only a single semaphore is locked.

The lock should be released as soon as possible after finishing

the command sequence.

INPUTS

I2CAddress - device address for which a lock is desired

or -1 for any.

RESULTS

The call should return a nonzero value for success or FALSE (0)

in case of an error.

BUGS

SEE ALSO

libraries/i2cclass.h, libraries/i2csensors.h, i2c_Release()

i2csensors.library/i2c_Release             i2csensors.library/i2c_Release

NAME

i2c_Release - unlock i2c bus after a sequence of read/write operations

SYNOPSIS

RESULT = i2c_Release( i2cAddress )

D0                    D0

LONG i2c_Release(LONG);

FUNCTION

Free I2C bus for other callers.

This function may not be called from interrupt code.

DESCRIPTION

One potential issue with the i2c.library is that competing

tasks may interfere with each other's read/write operations

on the bus, especially when a task switch occurs after the

prologue of an operation.

This function unlocks the bus such that other tasks may

carry out their I2C functionality. It is vital that

i2c_Release() is called as soon as the current read/write

operation sequence is done.

INPUTS

I2CAddress - device address for which a lock was obtained

RESULTS

The call should return just TRUE (1) for success or FALSE (0)

in case of an error.

BUGS

SEE ALSO

libraries/i2cclass.h, libraries/i2csensors.h, i2c_Obtain()

i2csensors.library/i2c_Send             i2csensors.library/i2c_Send

NAME

i2c_Send - send a number of bytes to I2C device

SYNOPSIS

RESULT = i2c_Send( I2CAddress, nbytes, senddata )

D0                 D0          D1      A1

LONG i2c_Send(UBYTE, UWORD, UBYTE *);

FUNCTION

Send 1-n bytes to an I2C address

This function may not be called from interrupt code.

DESCRIPTION

This is a convenience function that just passes the call

through to i2c.library. As long as the functionality in

this lib is sufficient for a task at hand, i2c.library

doesn't need to be opened manually when using this

library, anyway.

Hence, conventions and return values are the same

as with i2c.library.

Please ensure exclusive device access prior to this

call by means of i2c_Obtain() and release the device

when done with the read/write sequence.

INPUTS

I2CAddress - device address to send to

nbytes     - number of data bytes to write

senddata   - pointer to data bytes

RESULTS

result mask, same as i2c.library

BUGS

SEE ALSO

libraries/i2cclass.h, libraries/i2csensors.h, libraries/i2c.h, SendI2C()

i2csensors.library/i2c_Receive          i2csensors.library/i2c_Receive

NAME

i2c_Receive - receive a number of bytes from I2C device

SYNOPSIS

RESULT = i2c_Receive( I2CAddress, nbytes, senddata )

D0                    D0          D1      A1

LONG i2c_Receive(UBYTE, UWORD, UBYTE *);

FUNCTION

Receive 1...n bytes from an I2C address

This function may not be called from interrupt code.

DESCRIPTION

This is a convenience function that just passes the call

through to i2c.library. As long as the functionality in

this lib is sufficient for a task at hand, i2c.library

doesn't need to be opened manually when using this

library, anyway.

Hence, conventions and return values are the same

as with i2c.library.

Please ensure exclusive device access prior to this

call by means of i2c_Obtain() and release the device

when done with the read/write sequence.

INPUTS

I2CAddress - device address to send to

nbytes     - number of data bytes to write

senddata   - pointer to data bytes

RESULTS

result mask, same as i2c.library

BUGS

SEE ALSO

libraries/i2cclass.h, libraries/i2csensors.h, libraries/i2c.h, ReceiveI2C()

i2csensors.library/--------------Introduction----------------

This library is to be seen as add-on to existing hardware-software combinations

regarding I2C bus implementations on the Amiga. There exists a number of

different phyical approaches to I2C on the Amiga. These range from parallel and

serial port bitbang approaches, clockport interface chips up to Zorro II card

implementations. All these different ways of setting up an I2C interface require

appropriate hardware abstraction support.

An established quasi-standard is the I2C.library. Depending on the underlying

hardware, the appropriate i2c.library variant is to be installed and tested

first.

While the I2C.library provides the necessary means for hardware abstraction

and I2C compliant I/O, it was seen to lack a convenient interface to individual

devices. The i2csensors.library is one proposal to simplify sensor data access

for both developers and users.

The term simplify is relative when it comes to I2C. This bus was intended for

a limited number of simultaneusly active devices in a closed/fixed environment.

It doesn't come with luxuries like device classes and alongside them generic

capabilities most of us came to take for granted with modern designs like USB.

Moreover, it is known that several I2C capable chips on the market have the same

I2C Address. On top of these elaborations, many sensor chips are configurable

to provide different measurements as needed by the implementing party.

In order to provide a level of flexibility for the user, the functionality of

this library is configured by text files. This way, the supported hardware is

not limited to a small subset of available chips. Also, no programming

proficiency is required to extend the range of supported devices.

i2csensors.library/--------------Installation----------------

An important prerequisite is to test the I2C hard- and software layer before

i2csensors.library is set up. Please make sure that the hardware is installed

properly and the appropriate i2c.library is present in "LIBS:".

The i2clib40 distribution on Aminet contains a helpful utility called "I2CScan".

That tool scans the I2C bus for active devices. As soon as the installed devices

are detected at their assigned I2C addresses, you are ready for the next steps.

i2csensors.library consists of two parts. The library itself belongs to "LIBS:".

The second component are the sensors definitions. The active definitions

reside in "DEVS:Sensors/". It is suggested to move unneeded definitions into

"SYS:Storage/Sensors/".

Please move the definition(s) for the chips connected to your I2C bus into

"DEVS:Sensors/".

Once hardware, i2c.library, i2csensors.library and  configuration files are

in place, a test can be conducted. The distribution of i2csensors.library

features a small test and example program, called "simplesensors". Just run it

from CLI/Shell and it will display the readings for the defined sensors.

i2csensors.library/--------------Configuration---------------

The configuration is accomplished using text files. They follow the

common "KEY=VALUE" pattern. The configuration distinguishes between

devices and their individual sensors or measurement capabilities.

A device is defined by a string enclosed in square brackets. White

spaces are not allowed. The opening bracket must be the first character

in a line.

Comments also start at the first character in a row. The designation

for comments is "#" like in un*x shells.

Example contents of the configuration file Devs:Sensors/DS3231.cfg

# detailed description of keywords and parameters follows below

[DS3231]

I2CADDRESS = $d0

INIT       = 0e3c

TYPE       = TEMP

READPRE    = 11

READBYTES  = 2

BITOFFSET  = 1

NUMBITS    = 9

MUL        = 0.25

UNIT       = C

SIGNBIT    = 0

It's up to the user to either provide multiple configuration files

in DEVS:Sensors/ or to put everything in one file, separated by

respective [_CHOSEN_DEVICE_HEADER_NAME_] headers.

i2csensors.library/---------Configuration_keywords-----------

I2CADDRESS = I2C device address (once per device), either given in decimal

or hexadecimal notation (example: 208 or 0xd0 or $d0c are equivalent)

MANDATORY

INIT       = hexadecimal device init string, no whitespaces allowed,

hardware dependent

OPTIONAL

TYPE       = type of sensor, valid argument strings

VOLTAGE, CURRENT, TEMP, FAN, PRESSURE

MANDATORY

READPRE    = hexadecimal string to be sent to the device in front of the

actual sensor reading phase

OPTIONAL

READBYTES  = number of bytes to read (1-255)

MANDATORY

BITOFFSET  = first bit to extract, starting with index 0 for the most

significant bit of the first byte (0-255, default 0)

OPTIONAL

NUMBITS    = number of bits to extract for the absolute part, starting at

BITOFFSET (permitted range 1-31, default READBYTES*8)

OPTIONAL

SIGNBIT    = bit to extract as negative sign indicator, starting with index 0 for

the most significant bit of the first byte (0-255, default: unsigned number)

Two`s complement is applied, i.e. the sign is prepended to the

number and replicated to all higher order bits up to the MSB.

The sign bit is independent of NUMBITS, i.e. in addition to the

absolute part.

OPTIONAL

EXTRACTMASK= right aligned mask (NUMBITS bits) which designate which bits from

the extracted bits are kept (1) or skipped (0). This option is

useful for cases where there is extraneuos information between

sensor data bits

WAKEUP     = some sensors are in a relatively deep sleep mode (e.g. AM2320) and

need waking up before a read cycle can be performed, so if the

argument is 1, a dummy read/write is performed before the actual

sensor readout

UNIT       = string for the unit (V, A, K, ...)

So far, Integer numbers can be extracted from the values, depending on

application either unsigned or signed. Many chips provide fractional integer

readings. The i2csensors.library implements a linear model for sensor reading

correction and normalization. The formula is (int_value_read+ADD)*MUL

MUL        = fractional number in decimal notation (mantissa-exponent values

are not implemented as of this writing) for the multiplier

(default: 1)

OPTIONAL

ADD        = fractional number in decimal notation (mantissa-exponent values

are not implemented as of this writing) for the bias (default: 0)

OPTIONAL

There are cases where values are to be handled reciprocal, e.g. fan RPM

calculation from tick counters. The directive for such situations is DIVBY.

The formula in that case is MUL/(int_value_read+ADD).

DIVBY      = flag, denoting that the MUL constant is to be divided by the

sensor value (default: 0)

OPTIONAL

Some recent sensors don't fit the simple linear model. One example is the Bosch

BMP280. In order to deal with special requirements, a number of routines have

been implemented to support higher order adjustments. Such chips are announced

by the "CUSTOM" keyword. The downside of this approach is that special handler

code has to be provided by the library.

CUSTOM     = BMP280P or BMP280T